MT7140- Cliffside-

Location: Coordinates for tower: DM- 48.445339, -115.652958 
Elevation: 2126 ft.

We are looking for a 190’ Self Support tower for 4 carriers. 					     Below is the loading Materials to be provided include:

Complete tower steel and hardware
Anchor bolts and templates
Construction step bolts 
Climbing step bolts 
One (1) waveguide support ladder (to support twelve (12) initial lines)

Safety cable kit and leg brackets without harness
One (1) 2' x 5/8" lightning rod copper clad 							         
Certified tower profile drawing
Certified tower profile and foundation drawings
Final erection drawings

Tower Structures

  - Structure and foundation designed in accordance with Minimum ANSI/TIA 222 standards currently in effect, currently the “H” Code.

  - Lighting system in accordance with FAA requirements

  - Safety climbing cable defined by the client as mandated in the current ANSI/TIA standards, currently the “H” Code.

  - TIA standard grounding system provides a maximum of 5 ohms of resistance.

  - (1) 2’ Minimum copper clad lighting rod

  - Antenna design loading as follows:

    - Tower must have capacity for four (4) total broadband wireless (BBE) tenants defined as follows:

      - Top of tower taking 25,000 sq. in. of wind loading, including the antenna mount or platform

      - Top minus 10’ elevation taking 20,000 sq. in. of wind loading, including the antenna mount or platform.

      - Top minus 20’ elevation taking 20,000 sq. in. of wind loading, including the antenna mount or platform.

      - Top minus 30’ elevation taking 20,000 sq. in. of wind loading, including the antenna mount or platform.

    - Qty (1) 4’ solid MW dishes mounted at the top of the tower minus 40’ elevation.

    - Qty (1) 4’ solid MW dishes mounted at the top minus 50’ elevation.

    - Qty (1) 4’ solid MW dishes mounted at the top minus 60’ elevation.

    - Qty (1) 4’ solid MW dishes mounted at the top minus 70’ elevation.